ورق پوسته

توسط:

استاد درس:

###### فهرست مطالب

عنوان	صفحه

پروژه 1-........	1

پروژه 2-......................................................................................................................................... 9

# پروژه 1-

تمرین پروژه یک - مطلوب است محاسبه تابع خیز دریک ورق همسانکرد مستطیلی شکل که مطابق شكل زير بار کذاری شده است. همچنین مقدار عددی خیز بدست آمده به روش تحلیلی در زیر محل بارگذاری با روش اجزا محدود برای سه شدت بارگذاری مختلف متمرکز مورد مقایسه قرار کیرد. هر شدت باید دو برابر مرحله قبلی باشد. (P و2 Pو 4P) تمامی مقادیری كه در براى مسئله نياز هستند به اختيار دانشجو است (ضخامت، جنس، ابعاد ورق و محل و مقادير شدت باركذاری P و .... ) نتايج عددی و تحليلی بايد دريک جدول مقايسه وجمع بندی شوند.

فرض روش لوی (Levy Method)

خیز ورق به صورت سری فوریه:

wxy=m=1NYmxsinmπyb

صلبیت خمشی ورق

D=Eh3121-ν2

بسط فوریه بار متمرکز

qmx=2Pbsinmπy0bδx-x0

معادله دیفرانسیل هر مود

با تعریف:

βm=mπb

داریم:

Ym''''-2βm2Ym''+βm4Ym=qmxD

پاسخ عمومی معادله

Ymx=A1cosh⁡βmx+A2xsinh⁡βmx+A3sinh⁡βmx+A4xcosh⁡βmx

شرایط مرزی و پیوستگی

۱) لبه گیردار x=0

Ym0=0
Ym'0=0

۲) پیوستگی در محل بار x=x0

YIx0=YIIx0
YI'x0=YII'x0
YI''x0=YII''x0

شرط پرش نیروی برشی:

DYII'''x0-YI'''x0=2Pbsinmπy0b

۳) لبه آزاد x=a

شرط گشتاور خمشی صفر:

Ym''a-νβm2Yma=0

شرط نیروی برشی مؤثر صفر:

Ym'''a-2-νβm2Ym'a=0

شکل خیز در متلب

خروجی متلب:

Material: Titanium alloy (Ti-6Al-4V) (E = 110.0 GPa, nu = 0.31)

Plate: a = 1.50 m, b = 1.00 m, h = 15.0 mm

Flexural rigidity D = 34226.684 N.m

Load point: (x0, y0) = (0.750, 0.500) m

Convergence of w(x0,y0) [unit load, P=1 N] with number of Fourier terms:

n_modes =  3   w = 0.000439 mm

n_modes =  5   w = 0.000442 mm

n_modes =  8   w = 0.000444 mm

n_modes = 10   w = 0.000444 mm

n_modes = 15   w = 0.000445 mm

n_modes = 20   w = 0.000445 mm

Deflection under the point load - analytical (Levy method) vs FE:

Load_case    P_N     w_analytical_mm    w_FE_mm

_________    ____    _______________    _______

"1P"       1500        0.66781          NaN

"2P"       3000         1.3356          NaN

"4P"       6000         2.6712          NaN

کد MATLAB:

%      x = 0  : CLAMPED               (C)

%      x = a  : FREE                  (F)

%      y = 0  : SIMPLY SUPPORTED      (S)

%      y = b  : SIMPLY SUPPORTED      (S)

%      w(x,y) = sum_{m=1}^{N}  Y_m(x) * sin(m*pi*y/b)

%      q_m(x) = (2P/b) * sin(m*pi*y0/b) * delta(x-x0)

%

%      Y_m'''' - 2*beta_m^2*Y_m'' + beta_m^4*Y_m = q_m(x)/D

%      Y_m(x) = A1*cosh(bx) + A2*x*sinh(bx) + A3*sinh(bx) + A4*x*cosh(bx)

%      Clamped  @ x=0  : Y(0)=0 , Y'(0)=0

%      Continuity @ x0 : w, w', w'' continuous

%      Load jump @ x0  : D*(Y_II'''-Y_I''') = (2P/b)*sin(m pi y0/b)

%      Free @ x=a      : M_x=0 , V_x (Kirchhoff shear)=0

clear; clc; close all;

digits(300);   % 300 significant decimal digits

a_plate = 1.5;        % plate length, clamped (x=0) to free (x=a) edge

b_plate = 1.0;        % plate width, between the two simply supported edges

h       = 0.015;      % plate thickness

material = 'Titanium alloy (Ti-6Al-4V)';

E   = 110e9;          % elastic modulus

nu  = 0.31;           % Poisson's ratio

x0 = a_plate/2;       % load location (x) - assumed at plate centre

y0 = b_plate/2;       % load location (y) - assumed at plate centre

P_base     = 1500;                    % base load magnitude

load_cases = [1 2 4] * P_base;        % P, 2P, 4P (assignment requirement)

n_modes = 20;          % number of Fourier (Levy) terms

grid_n  = 21;          % grid resolution for the deflected-shape plot only

D = E*h^3/(12*(1-nu^2));      % flexural rigidity of the plate          [N.m]

fprintf('Material: %s (E = %.1f GPa, nu = %.2f)\n', material, E/1e9, nu);

fprintf('Plate: a = %.2f m, b = %.2f m, h = %.1f mm\n', a_plate, b_plate, h*1000);

fprintf('Flexural rigidity D = %.3f N.m\n', D);

fprintf('Load point: (x0, y0) = (%.3f, %.3f) m\n\n', x0, y0);

av = vpa(a_plate); bv = vpa(b_plate);

x0v = vpa(x0);      y0v = vpa(y0);

nuv = vpa(nu);       Dv = vpa(D);

beta_all = cell(n_modes,1);

A_all    = cell(n_modes,1);

B_all    = cell(n_modes,1);

for m = 1:n_modes

[beta, Avec, Bvec] = solve_mode(m, av, bv, x0v, y0v, nuv, Dv);

beta_all{m} = beta;

A_all{m}    = Avec;

B_all{m}    = Bvec;

end

fprintf('Convergence of w(x0,y0) [unit load, P=1 N] with number of Fourier terms:\n');

running = vpa(0);

check_at = [3 5 8 10 15 20];

for m = 1:n_modes

running = running + Y_at(x0v, x0v, beta_all{m}, A_all{m}, B_all{m}) * sin(beta_all{m}*y0v);

if any(m == check_at)

fprintf('  n_modes = %2d   w = %.6f mm\n', m, double(running)*1000);

end

end

w_unit_x0y0 = double(running);    % deflection at load point, per unit N of load

fprintf('\n');

Load_case       = strings(numel(load_cases),1);

P_N              = zeros(numel(load_cases),1);

w_analytical_mm  = zeros(numel(load_cases),1);

w_FE_mm          = nan(numel(load_cases),1);     % <-- fill in from Abaqus

for i = 1:numel(load_cases)

P = load_cases(i);

Load_case(i)      = sprintf('%.0fP', P/P_base);

P_N(i)            = P;

w_analytical_mm(i) = w_unit_x0y0 * P * 1000;   % m -> mm

end

T = table(Load_case, P_N, w_analytical_mm, w_FE_mm);

disp('Deflection under the point load - analytical (Levy method) vs FE:');

disp(T)

writetable(T, 'plate_deflection_results_titanium.csv');

xs = linspace(0, a_plate, grid_n);

ys = linspace(0, b_plate, grid_n);

W  = zeros(grid_n, grid_n);

for m = 1:n_modes

beta = beta_all{m}; Avec = A_all{m}; Bvec = B_all{m};

Yx = zeros(1, grid_n);

for i = 1:grid_n

Yx(i) = double(Y_at(vpa(xs(i)), x0v, beta, Avec, Bvec));

end

sinY = sin(double(beta) * ys);

W = W + Yx.' * sinY;             % outer product, accumulate mode contribution

end

W_mm = W * P_base * 1000;            % deflection [mm] for the base load case

[Xg, Yg] = ndgrid(xs, ys);

figure('Color','w');

surf(Xg, Yg, -W_mm);                 % plotted as -W so downward deflection points down

shading interp; colorbar

xlabel('x (m)  [0 = clamped, a = free]');

ylabel('y (m)  [0,b = simply supported]');

zlabel('Deflection (mm)');

title(sprintf('Deflected shape, P = %.0f N (%s plate)', P_base, material));

fprintf('\nMax deflection on the plotted grid: %.4f mm (thickness = %.1f mm - ', ...

max(W_mm(:)), h*1000);

function v = phi_fun(x, beta)

c = cosh(beta*x); s = sinh(beta*x);

v = [c, x*s, s, x*c];

end

function v = dphi_fun(x, beta)

c = cosh(beta*x); s = sinh(beta*x);

v = [beta*s, s + beta*x*c, beta*c, c + beta*x*s];

end

function v = d2phi_fun(x, beta)

c = cosh(beta*x); s = sinh(beta*x);

v = [beta^2*c, 2*beta*c + beta^2*x*s, beta^2*s, 2*beta*s + beta^2*x*c];

end

function v = d3phi_fun(x, beta)

c = cosh(beta*x); s = sinh(beta*x);

v = [beta^3*s, 3*beta^2*s + beta^3*x*c, beta^3*c, 3*beta^2*c + beta^3*x*s];

end

function [beta, Avec, Bvec] = solve_mode(m, a, b, x0, y0, nu, D)

beta = vpa(pi)*m/b;

Fm   = (2/b) * sin(vpa(m)*vpa(pi)*y0/b);   % forcing amplitude, unit load P=1N

M   = vpa(zeros(8,8));

rhs = vpa(zeros(8,1));

% Row 1: clamped, Y_I(0) = 0

row = phi_fun(vpa(0), beta);

M(1,1:4) = row;

% Row 2: clamped, Y_I'(0) = 0

row = dphi_fun(vpa(0), beta);

M(2,1:4) = row;

% Row 3: continuity of w at x0

rI = phi_fun(x0, beta);

M(3,1:4) = rI;  M(3,5:8) = -rI;

% Row 4: continuity of w' at x0

rI = dphi_fun(x0, beta);

M(4,1:4) = rI;  M(4,5:8) = -rI;

% Row 5: continuity of w'' at x0

rI = d2phi_fun(x0, beta);

M(5,1:4) = rI;  M(5,5:8) = -rI;

% Row 6: jump in w''' at x0  ->  D*(Y_II'''-Y_I''') = Fm

rI = d3phi_fun(x0, beta);

M(6,1:4) = -rI; M(6,5:8) = rI;

rhs(6) = Fm/D;

% Row 7: free edge moment, Y_II''(a) - nu*beta^2*Y_II(a) = 0

r2 = d2phi_fun(a, beta); r0 = phi_fun(a, beta);

M(7,5:8) = r2 - nu*beta^2*r0;

% Row 8: free edge Kirchhoff shear, Y_II'''(a) - (2-nu)*beta^2*Y_II'(a) = 0

r3 = d3phi_fun(a, beta); r1 = dphi_fun(a, beta);

M(8,5:8) = r3 - (2-nu)*beta^2*r1;

sol  = M \ rhs;

Avec = sol(1:4);

Bvec = sol(5:8);

end

function val = Y_at(x, x0, beta, Avec, Bvec)

% Evaluate the (unit-load) Y_m(x) piecewise function.

if x <= x0

coeffs = Avec;

else

coeffs = Bvec;

end

v = phi_fun(x, beta);

val = sum(coeffs.' .* v);

end

# پروژه 2-

تمرین پروژه دو - مطلوب است تحلیل تنش غشایی که یک پوسته دوار که حاصل از دوران يک منحنی سهمی شکل است. پوسته صرفا تحت وزن خود قرار دارد و تحلیل غشایی را در محلی دور از لبه های تكیه کاهی برای تنشهای نصف النهاری و عرض از مبدا انجام دهید. نتایج را با روش عددی (نرم افزار اجزا محدود) در محل یکسان مقایسه کنید و در جدولی جمع بندی کنید. ( همه اطلاعات داده نشده در مسئله به اختیار دانشجوست)

مدل‌سازی هندسه

معادله هندسه گنبد و شیب آن:

z=ar2
tanϕ=2ar

روابط مثلثاتی:

sinϕ=2ar1+4a2r2
cosϕ=11+4a2r2

شعاع انحنای نصف‌النهاری:

R1=1+4a2r23/22a

شعاع انحنای محیطی:

R2=rsinϕ=1+4a2r22a

مدل‌سازی بارگذاری (وزن خودی)

وزن کل بخش فوقانی پوسته تا شعاع r:

Wr=πq06a21+4a2r23/2-1

معادلات تعادل غشایی

نیروی نصف‌النهاری:

Nϕ=-Wr2πrsinϕ

نیروی محیطی:

NϕR1+NθR2=-q0cosϕ

بنابراین:

Nθ=-R2q0cosϕ-NϕR2R1

مقادیر در تاج گنبد (Apex)

Nϕ0=Nθ0=-q04a

محاسبه تنش‌ها

σϕ=Nϕt
σθ=Nθt

نمودار های ایجاد شده توسط متلب

خروجی کد متلب

Parabola coefficient   a  = 0.32000  (1/m)

Self-weight per area   q0 = 3.5316 kN/m^2

Apex values (r=0):   N_phi = N_theta = -2.7591 kN/m

--- Comparison point (away from support edge, r/R = 0.60) ---

r = 3.000 m ,  phi = 62.49 deg

N_phi   (membrane theory) =   -9.878 kN/m  ->  sigma_phi   =   -65.85 kPa

N_theta (membrane theory) =   -3.410 kN/m  ->  sigma_theta =   -22.74 kPa

کد MATLAB:

%  analytically for the meridian  f(r) = a*r^2.

%  Vertical equilibrium of the cap above radius r:

%  N_phi   = - W(r) / (2*pi*r*sin(phi))

%  W(r)    = total self-weight of the shell above the parallel at r

%  Equilibrium normal to the shell surface:

%  N_phi/R1 + N_theta/R2 = -Z ,      Z = q0*cos(phi)

%  N_theta = -R2*q0*cos(phi) - N_phi*(R2/R1)

clear; clc; close all;

R      = 5.0;

H      = 8.0;

t      = 0.15;

gamma  = 23.544;

r_edge_ratio = 0.90;

r_eval_ratio = 0.60;

n_pts  = 500;

a  = H / R^2;

q0 = gamma * t;

fprintf('Parabola coefficient   a  = %.5f  (1/m)\n', a);

fprintf('Self-weight per area   q0 = %.4f kN/m^2\n\n', q0);

r_min = 1e-4 * R;

r     = linspace(r_min, r_edge_ratio*R, n_pts).';

%     f(r)  = a*r^2

%     f'(r) = 2*a*r = tan(phi)

fp   = 2*a*r;

phi  = atan(fp);

sphi = fp ./ sqrt(1 + fp.^2);             % sin(phi)

cphi = 1  ./ sqrt(1 + fp.^2);             % cos(phi)

R1 = (1 + fp.^2).^1.5 / (2*a);            % meridional radius of curvature

R2 = r ./ sphi;                           % second principal radius

W = (pi*q0/(6*a^2)) .* ( (1 + 4*a^2*r.^2).^1.5 - 1 );

N_phi   = - W ./ (2*pi*r.*sphi);

N_theta = - R2.*q0.*cphi - N_phi.*(R2./R1);

% analytic apex limit (r -> 0), used only as a reference / sanity check

N_phi0   = -q0/(4*a);

N_theta0 = N_phi0;                        % N_phi = N_theta at the apex

fprintf('Apex values (r=0):   N_phi = N_theta = %.4f kN/m\n\n', N_phi0);

sigma_phi   = N_phi   / t;     % [kN/m^2 = kPa]

sigma_theta = N_theta / t;     % [kN/m^2 = kPa]

figure('Color','w');

subplot(2,1,1)

plot(r/R, N_phi, 'b-', 'LineWidth', 1.6); hold on

plot(r/R, N_theta, 'r-', 'LineWidth', 1.6);

xlabel('r / R'); ylabel('Membrane force (kN/m)');

legend('N_\phi (meridional)','N_\theta (hoop)','Location','best');

title('Membrane forces along the meridian - self-weight only'); grid on

subplot(2,1,2)

plot(rad2deg(phi), N_phi, 'b-', 'LineWidth', 1.6); hold on

plot(rad2deg(phi), N_theta, 'r-', 'LineWidth', 1.6);

xlabel('\phi  (deg)'); ylabel('Membrane force (kN/m)');

legend('N_\phi','N_\theta','Location','best'); grid on

ratios_report = (0:0.1:r_edge_ratio).';

r_report      = ratios_report * R;

phi_r  = atan(2*a*r_report);

sphi_r = sin(phi_r);

cphi_r = cos(phi_r);

R1_r   = (1 + (2*a*r_report).^2).^1.5 / (2*a);

R2_r   = r_report ./ max(sphi_r, eps);

W_r    = (pi*q0/(6*a^2)) .* ((1 + 4*a^2*r_report.^2).^1.5 - 1);

Nphi_r      = -W_r ./ (2*pi*max(r_report,eps).*max(sphi_r,eps));

Nphi_r(1)   = N_phi0;                      % use analytic apex value

Ntheta_r    = -R2_r.*q0.*cphi_r - Nphi_r.*(R2_r./R1_r);

Ntheta_r(1) = N_theta0;

sigphi_r   = Nphi_r   / t;

sigtheta_r = Ntheta_r / t;

T = table(ratios_report, r_report, rad2deg(phi_r), Nphi_r, Ntheta_r, ...

sigphi_r, sigtheta_r, ...

'VariableNames', {'r_over_R','r_m','phi_deg','N_phi_kNm','N_theta_kNm', ...

'sigma_phi_kPa','sigma_theta_kPa'});

disp('Membrane theory results along the meridian:');

disp(T)

writetable(T, 'membrane_theory_results.csv');

r_eval    = r_eval_ratio * R;

phi_eval  = atan(2*a*r_eval);

sphi_eval = sin(phi_eval);

cphi_eval = cos(phi_eval);

R1_eval   = (1 + (2*a*r_eval)^2)^1.5 / (2*a);

R2_eval   = r_eval / sphi_eval;

W_eval    = (pi*q0/(6*a^2)) * ((1 + 4*a^2*r_eval^2)^1.5 - 1);

Nphi_eval     = -W_eval / (2*pi*r_eval*sphi_eval);

Ntheta_eval   = -R2_eval*q0*cphi_eval - Nphi_eval*(R2_eval/R1_eval);

sigphi_eval   = Nphi_eval   / t;

sigtheta_eval = Ntheta_eval / t;

fprintf('\n--- Comparison point (away from support edge, r/R = %.2f) ---\n', r_eval_ratio);

fprintf('r = %.3f m ,  phi = %.2f deg\n', r_eval, rad2deg(phi_eval));

fprintf('N_phi   (membrane theory) = %8.3f kN/m  ->  sigma_phi   = %8.2f kPa\n', Nphi_eval, sigphi_eval);

fprintf('N_theta (membrane theory) = %8.3f kN/m  ->  sigma_theta = %8.2f kPa\n', Ntheta_eval, sigtheta_eval);

% --- paste your FE (finite element) results here to compare ------------

N_phi_FE   = [];   % <-- fill in from SAP2000 / ANSYS / ABAQUS

N_theta_FE = [];

if ~isempty(N_phi_FE) && ~isempty(N_theta_FE)

fprintf('\n--- FE comparison ---\n');

fprintf('N_phi:   FE = %.3f kN/m , difference = %.2f %%\n', ...

N_phi_FE, 100*(N_phi_FE - Nphi_eval)/Nphi_eval);

fprintf('N_theta: FE = %.3f kN/m , difference = %.2f %%\n', ...

N_theta_FE, 100*(N_theta_FE - Ntheta_eval)/Ntheta_eval);

end